- Small size, high reliability, wide operating wavelength range

- Application for EPON, GPON, FTTx networks

- G657A1 fiber or customer specified

- Splitter ration from 1:2 to 1:64

Part number

| PRPLC-S01-XXX-XXX | Planar light wave circuit splitter, insert type |
| --- | --- |
| 1 2 |  |
| *1 indicate splitter capacity:102=1X2;202=2X2;104=1X4;204=2X4;108=1X8;208=2X8;116=1X16;216=2X16… |  |
| *2 indicate the splitter adapter style: FC=FC; FCA=FC/APC; LC=LC; LCA=LC/APC; SC=SC; SCA=SC/APC… |  |

Performance

| Parameters | Specification |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating wavelength (nm) | 1260-1650 |  |  |  |  |  |
| Type | 1X2 | 1X4 | 1X8 | 1X16 | 1X32 | 1X64 |
| Insertion loss (dB) | ≤3.8 | ≤7.1 | ≤10.2 | ≤13.3 | ≤16.5 | ≤19.8 |
| Loss uniformity (dB) | ≤0.3 | ≤0.3 | ≤0.3 | ≤0.5 | ≤0.5 | ≤1.0 |
| PDL (dB) | ≤0.08 | ≤0.08 | ≤0.08 | ≤0.1 | ≤0.1 | ≤0.2 |
| Type | 2X2 | 2X4 | 2X8 | 2X16 | 2X32 | 2X64 |
| Insertion loss (dB) | ≤3.8 | ≤7.1 | ≤10.2 | ≤13.3 | ≤16.5 | ≤20.1 |
| Loss uniformity (dB) | ≤0.5 | ≤0.5 | ≤0.5 | ≤0.8 | ≤0.8 | ≤1.5 |
| PDL (dB) | ≤0.1 | ≤0.1 | ≤0.1 | ≤0.15 | ≤0.15 | ≤0.3 |
| Return loss (dB) | ≥55 |  |  |  |  |  |
| Directivity (dB) | ≥55 |  |  |  |  |  |
| Wavelength dependent Loss (dB) | ≤1.0 |  |  |  |  |  |
| Pigtail length (m) | 1.0m or customer specified |  |  |  |  |  |
| Fiber type | ITU G657A1 or customer specified |  |  |  |  |  |
| Operating temperature (℃) | -40℃ to +85℃ |  |  |  |  |  |
| Storage Temperature (℃) | -40℃ to +85℃ |  |  |  |  |  |

While the information has been carefully compiled to the best of our knowledge, nothing is intended as representation or warranty on our part and no statement herein shall be construed as recommendation to infringe existing patents. In the effort to improve our products, we reserve the right to make changes judged to be necessary.